ADD SPRAYER MAINTENANCE TO SPRING CLEANING TASKS

Source: Tim Stombaugh

Remember to include sprayer maintenance in your farm spring cleaning tasks. It will ensure that your spray equipment is ready for the planting season, as well as save you time and money.

Taking care of sprayer maintenance prior to the hectic growing season can prevent time-consuming equipment breakdowns, higher chemical costs, reduced pesticide effectiveness and potential crop damage.

Poorly maintained sprayer equipment can cause variations in pesticide application rates. These variations can lead to ineffective pest control and potential crop injury, resulting in higher chemical costs and reduced profits. Precise pesticide application is even more important with the highly active agricultural chemicals we have on the market today.

Make the following a part of your spray equipment spring cleaning agenda:

* Rinse out the sprayer to remove any dirt that accumulated over the winter.

* Check the pump for excessive wear and to be sure it is operating at full capacity.

* Inspect sprayer lines for leaks.

* Visually inspect nozzles for excessive wear, corrosion or damage.

* Measure the output from each nozzle to ensure uniform application.

* Visually inspect spray from each nozzle to find any inconsistent patterns resulting from worn or damaged nozzles.

* Clean filter screens and replace worn ones.

* Check the agitator for proper turbulence so specific formulations will be mixed well.

* On a driveway or other appropriate site, use water to check spray patterns for proper overlap.

* Inspect all electrical connections on sprayer controllers to find corroded or loose wires.

For more information on profitable farm management practices, contact your (COUNTY NAME) Cooperative Extension Service.

Educational programs of the Cooperative Extension Service serve all people regardless of race, color, age, sex, religion, disability or national origin.

-30-