It’s always time to think about farm safety

Source:  Mark Purschwitz, extension professor and agricultural safety and health specialist

The recent death of a Lincoln County farmer who was trapped beneath the tailgate of a round hay baler is a stark reminder of the need to use safety locking devices. Media reports stated that the farmer was working inside the baler to cut out a clog when the tailgate closed behind him, trapping and ultimately killing him.

It’s a good time for all farmers to evaluate their own safety methods around machinery so they can avoid preventable accidents on their farms.

Whenever you work around or beneath farm machines or machine components held up by hydraulics, you should always engage the locking devices supplied by the manufacturer. Hydraulic systems can leak or rupture. Even someone helping you might hit the wrong lever, so it is important that you use the locking device every time, without exception.  Check the owner's manual to be sure you know how to use it.

Combine headers, skid-steer loader arms and round baler tailgates are just a few examples of machines that typically have locking devices, specifically for the purpose of safely performing maintenance or repairs. Such devices generally consist of mechanical supports such as steel pins or channels that prevent a hydraulic cylinder from retracting and lowering the machine, but since every machine is different, it is important to consult the owner's manual.

Older machines may not have safety locks, so you must have wood blocks or jack stands to hold up the machine - or angle iron or steel channel strapped to the hydraulic cylinder rod to prevent retraction, while someone is beneath it.

Regardless of a machine’s age, you never want to trust your life to anything supported by a hydraulic system.  Machines fail, and they do not care who you are.

For more information on farm safety, contact the (COUNTY NAME) Cooperative Extension Service.

Educational programs of the Kentucky Cooperative Extension Service serve all people regardless of race, color, age, sex, religion, disability or national origin.

-30-